# Just ‘Cuz Antweight Arena Construction Guide

## Click Here for the full Bill of Materials/Parts List

## Important Notes:

I used ¼-20 economy T nuts, which act as roll-in T nuts with 15 series profiles when inserted with the boss side up. This makes construction much easier while also saving a few bucks compared to ⅜-16 or 8mm hardware. Because I was grabbing from a pile of profiles, I actually confused all of the wide profiles which were 40mm metric with the 1.5” profile, as they are extremely similar, but ever so slightly wider (1.574”). The economy ¼-20 T nuts WILL NOT roll into a 40 series profile, so I strongly recommend using only 15 series as it makes things far easier/cheaper vs buying all roll-in or drop-in T nuts.

I used Send Cut Send to cut 0.119” thick steel plates for all joints as they generously sponsored my build and that was free, but it will also be cheaper for you to do the same than buying plates from 80/20. The laser DXF files are included at the Full CAD Assembly link here and down below. You can also go to this Autodesk repository link to download in any CAD format.

I do not recommend any larger than 3lb bots be run with 6mm polycarbonate. 6mm might not even be adequate for beetleweights, especially with any bots that have a weight bonus and extra heavy weapons. This is why I am labelling this an antweight arena, not a beetleweight arena. You could potentially use 8mm polycarb instead of 6mm, though this would be considerably more expensive. Look down at the bottom for more info.

## Safety:

Combat robots are dangerous, and anyone who builds them should know the risks. Building an arena and hosting an event creates even more numerous opportunities for things to go wrong or for people to get hurt, especially if the arena itself is not built appropriately. I know many people may be excited to throw an arena together and start fighting robots right away, but I strongly discourage anyone who isn’t fully comfortable with this sort of construction from attempting to do so. Not only is building this a sizable expense, but it does take a decent amount of work to get right, and cutting corners in the wrong ways can endanger yourself and others.

While I did put forth some effort to make this assembly guide as clear and complete as I could, and I put serious effort into making the design robust, I cannot guarantee in any way that even if these instructions are followed exactly, let alone if any steps are missed, that the arena will fully withstand any and all events that may occur during a combat robot fight. Common sense and best practices should be followed to fill in any details I may have missed.

Please read the disclaimer in the project folder as it applies to all information provided herein.

## Tools and Equipment Needed

Necessary:

- Cordless drill

- Sharp Drill Bits for Steel and Wood

- Measurement tools (tape measure/Ruler)

- Sharpies/Pencils

- Hacksaw

- Files & Sandpaper

- Portable jigsaw or circular saw* (if sizing wood or plastic panels yourself)

Optional Extras:

- Metal cutting bandsaw, circular saw, miter saw, or chop saw (if cutting profiles)

- Drill press

- Calipers

- Table saw (if sizing wood or plastic panels yourself)

- Paint roller/tray

- Truck or vehicle capable of hauling 4’x4’ sheets

The way I designed my arena, it should be able to be replicated with little more than a cordless drill, some measurement tools, a hacksaw, and hex drivers. The most involved part of the build will be drilling holes in steel kick plates, which would be greatly aided by the use of a drill press and quality drill bits. No CNC router or milling required. If you can’t access a table saw, the polycarbonate can be sourced and cut to size by the vendor, and home depot offers free or cheap panel cuts at most locations to cut down MDF or plywood sheets to the needed sizes (though they may not be perfect). 80/20 also offers the ability to purchase pre-cut lengths of the exact sizes you need for all profiles, although sourcing longer profiles and cutting them yourself should be a bit more affordable (and in my case all the profiles were freely available). Every single part used is available off the shelf from big box stores, online industrial stores, or normal web stores.

The build did run into a  few hiccups, however. Firstly, despite owning a CRV SUV, I wasn’t able to fit the 48x48” MDF subfloor in my car and still close the trunk, and was forced to use ratchet straps to hold the trunk door closed on the quick drive between home depot and the makerspace. Carrying and handling the MDF is really tricky with just one person as dropping it on an edge or corner will crush it easily, and these big squares are easily 50 pounds. This arena also obviously takes up a lot of space, and while it can collapse when disassembled, it is still somewhat annoying to reassemble since the economy T nuts won’t keep themselves in place. Fully assembled the arena will weigh approximately 270 pounds, with around half in just the base. The five 6mm ~2’x4’ polycarbonate panels are around 23 pounds each.

## Build Process - Click Here for the Full CAD Assembly

- Cut profiles to size

  - If they are not precut, make sure you end up with

    - 5X 45” long 1530 (wide) profiles

    - 4X 23” long 1515 profiles

    - 2X 45” long 1515 profiles

    - 2X 48” long 1515 profiles

- Cut MDF/Plywood to size

  - If they are not precut, make sure you end up with

    - You will need a 48”x48” subfloor (some wiggle room) and a 45”x45” floor (exact or smaller)

    - You may want to get/cut extra wood to replace the 45x45 floor when it is damaged

    - For the front/doorway you will need two 20” x 15” pieces and one 19.75” x 15” for the door. You might want to wait until the frame is together to cut these, as a snug fit may require some adjustments based on your exact frame.

  - Prep Work for Plywood or Hardwood Panels

    - If you want to stain, coat, paint, or otherwise prep the front/door surfaces, now is the time. You will need to drill and screw into these parts afterwards, so it may be worth following the next steps first with fresh wood, and once it all lines up, prep the wood as desired and reassemble.

    - Personally the latter was what I did. I used some wood stain and three layers of polycoat to protect the inner and outer surfaces of the plywood front and door.

    - The 45x45 floor can be added or removed at any time by lifting the assembled entire frame up off the subfloor, so this can be dealt with later if you choose.

- Cut Polycarbonate to size

  - If they are not precut, make sure you end up with

    - Three panels 20 ⅞” wide and two 21 ⅞” wide, all five should be 45 ⅞” long

    - If you are making your corner vertical profiles 24” tall, then you can use 5 exactly identical sheets all 21 ⅞” wide

    - All panels need to be about 0.879” (~⅞”) longer than the INSIDE width between frame members

    - I got mine pre-cut by Curbell Plastics, three 20.879” x 45.879” and two 21.879” x 45.879”

- Prepare Kick Plates

  - First, cut the kick plates such that you have four identical lengths all at the longest 45” minus twice the material thickness. For me this was 44.75, with ⅛” plates.

    - If using a circular saw, DO NOT use a steel cutting blade on aluminum. It will clog up the teeth and can ruin the blade.  Any blade marked as for ‘ferrous metals’ is for steel and similar materials only.

    - Metal cutting band saws will usually handle aluminum or steel fine. Wood cutting miter saws or bandsaws may also work well on aluminum, but not steel.

    - When in doubt, a hacksaw will be fine and safe, if slow

  - Next, drill holes to mount the plates. These holes do not need to be accurately located along the length of the plate, but should all be lined up 1.5” from the ‘bottom’ edge of the plate, so that they align with the higher T slot in the profile, offset 0.75” by the floor.

    - You can choose how many holes and their spacing. I did 9 holes, ~2.5” from each side, 5” apart.

    - I found the best way to do this with steel was to get a sharp 3/16” drill bit to pilot each hole, and then drill to full size with a 7/64” bit, both of which I got from home depot. Cobalt, or TiN-coated HSS bits should be fine.

  - The kick plates can be made of any suitable material or thickness really, but should be 2” or taller to suit your needs. I used ⅛” x 3” mild steel.

  - Wood or plastic could be used but will need to be far thicker and consume more of the internal space, with lengths needing to be adjusted accordingly.

- Attach Castors to the Subfloor

  - You will need:

    - 48x48 Subfloor

    - 4X Castors

    - 16X sets of wood screws and washers

    - Pencil to mark holes

    - Something to use as a spacer, about 1.5-2” wide (L shaped plates work well)

    - Cordless Drill and pilot drill bit to match the closen screws (⅛” bit for #8 screws)

  - First, lay the MDF flat on the floor or a table. Use the spacer to position the castor off each edge in a corner of the MDF

  - Use a pencil or something to trace the inside of the castor plate mounting holes, while holding the castor firmly in place

  - Repeat b and c on the other four corners

  - Drill all the way through all of the marked holes with your pilot drill bit

  - On each corner, line up the castor with the pilot holes, and insert a screw in each corner, loosely until all four are in and then tighten by hand. Avoid over tightening as MDF can be stripped easily.

    - If using a drill with an adjustable clutch to insert the screws, set it to its lowest setting before driving any screws into MDF. Slowly increase torque if that’s not enough to bottom them out, but final tighten should be by hand to be safe

  - If any screws are sticking through the MDF to the opposite side, now is a good time to cut the ends off with a dremel cutoff wheel or quality flush cutters and sand the surrounding wood flat. The floor needs to sit flush above these.

  - Set the subfloor castors on the floor in an open and fairly flat area. If possible, lock the wheels or lower the feet to keep the subfloor from rolling all over.

- Assemble the Frame!

  - There are a lot of T nuts and connecting plates holding this thing together. Because all of these connections rely on friction, it is essentially that once each part is aligned and squared up, all screws are tightened down fully. Loctite may not be necessary but may be beneficial.

    - Detailed instructions to follow are a bit overly specific about each step and the order of it all. When using T nuts that roll in or drop in, the order of assembly is not nearly as critical. However, if yours slide in from the sides, it is important to look at the full CAD and determine exactly how many nuts are needed in each channel before you tighten down screws that block the ends.

  - The basic order is - assemble complete ceiling, assemble a U shape from the frame pieces without the front wall/ door, with the three wall PC panels, then sit the ceiling atop this and attach it, loosening and re-tightening as needed to square up the frame.

  - After this, the wood for the front can be cut to snugly fit whatever your real-world frame size is.

  - Then, mark, pilot drill, and screw together the front wood panels at either side, line up the door and adjust it to fit, then line up, mark, drill, and attach the latch mechanism. That’s really it!

- Assemble the Ceiling

  - This will use

    - 1X 45” 1530 wide profile

    - 2X 45” 1515 profile

    - 2x 48” 1515 profile

    - 2x taller polycarbonate panels

    - 4X 4-hole L corner brackets

    - 2X rectangle 6-hole plates

  - First, try to lay the whole thing out flat on the 48x48 MDF or another big flat surface, making sure the cut lengths are correct and a square can be formed

    - Pull one 48” profile aside

    - Ensure the 45” long profiles are all parallel and they are overlapped by the 48” profile

  - Add the 4-hole L brackets to one corner between the 48” and 45” profiles. Add the two screws and T nuts on the 48” long pieces and hand tighten them, lining up the outer edges of the brackets and 45” long profiles by eye or feel

    - Repeat on the opposite corner of the same 48” profile, and if the brackets are square, tighten these four screws fully

  - Now add the two screws and nuts to the 45” profiles on each end and snug up (not tighten) both, ensuring the frame stays square

  - Now you should have three sides joined. Carefully slide a polycarbonate panel in within the profile slots until it reaches all the way back to the joined corners

  - Add the middle 1530 wide profile, and slide it back until the PC fits in the side groove there. It should be nearly centered

  - Add the two six-hole plates and all four nuts and screws in the wide profile on both sides, and hand tighten these

  - Then add the two nuts and screws to the 48” long profiles, and snug (not tighten) those as well

  - Slide in the second PC panel as before but this time until it reaches into the central wide profile slot

  - Slide the last 45” long profile against it and line it up overlapping with the corners of the 48” long pieces to complete the square frame

    - If this profile cannot go in far enough, the first PC panel may have bent down and not gone into the slot on the 48” long profile at the far end. Use a hex driver or long object to reach under thand push up in the middle of the panel while trying to slide it down further until it enters the slot. You may need to repeat this for the central wide profile too.

  - Now add the remaining 4 hole corner brackets, nuts, and screws, and join the entire ceiling together

  - Finally, ensuring corners are lined up and square, tighten everything down.

- Assemble the rear PC panel wall

  - This will use:

    - 1X 45” 1530 wide profile

    - 2X 23” 1515 profile

    - 1x shorter polycarbonate panel

    - 2X 4-hole L corner brackets

    - 2X rectangle 6-hole plates

  - Lay the profiles on the MDF or other large flat surface such that the 23” profiles overlap the 45” wide profile and meet at the same bottom edge

  - Add the 6 hole plates with 4 T nuts and 4 screws on one end of the wide profile, such that they line up with the outside edge of the 23” profile. Hand tighten these screws

    - Repeat on the opposite side

    - Note - these must not overlap the edge on the outside or it will be an issue when adding the side walls.

  - Add the two T nuts and screws to secure the 23” profiles on either end, again hand tight

  - If you have a square or 123 blocks or another flat object, use this to line up the 23” profiles with the bottom edge of the wide profile, then fully tighten all screws on one side

    - Repeat on the opposite side

    - You should now have a short rectangular U shape.

  - Carefully slide a polycarbonate panel into this shape (standing on end makes this easier as it won’t bend away from the bottom slot

  - Add the 4-hole L bracket on one side of the 23” profile such that two holes of the long side are on the profile and the short end sticks up 1.5” (a profile width)

  - Add the two nuts and screws to hand-tighten this in place

    - Repeat on the opposite side

- Assemble partial sidewalls

  - This will use:

    - 2X 45” 1530 wide profile

    - 2X 23” 1515 profile

    - 2x shorter polycarbonate panel

    - 2X rectangle 6-hole plate

  - This will involve making two L shapes with a wide 45” profile and a short 23” profile

  - Basically do Step 5b-5f again, but only on one side

    - These will be mirrors of each other so one becomes the front left corner and one becomes the front right corner. Both need plates and screws on the outside so make one left-hand corner and one right-hand corner

- Remove corner interferences

  - Due to the fact that the ceiling consists of two 48 and two 45 profiles, the 48” profile will intersect the top corners of any polycarbonate panels which protrude above the 23” profiles on that edge

  - To solve this there are two options:

    - Use a hacksaw/bandsaw to remove about a 0.5 inch square from the top corners of a shorter PC panel or two

    - Use a hacksaw/bandsaw to notch the aluminum profiles to clear the PC

  - I think option 1. is much easier so that’s what I did with a coping saw

  - Another important note: if the ceiling frame long profiles are aligned front to back, then you only need to do this to the rear PC panel, as I did. If they are aligned side-to-side, you need to do this to both side panels

- Assemble the U frame

  - This will use:

    - 1X 45” 1530 wide profile

    - 6X rectangle 6-hole plates

    - Complete rear wall

    - Two partial sidewalls

  - Pick a side. Add the 6 hole plates with 4 T nuts and 4 screws on one end of the wide profile, such that the last 2 holes protrude from the open end. Leave these screws somewhat loose.

    - Repeat on the opposite side

  - Bring the side wall pieces together with the rear wall frame, standing up to form 3 sides of the arena. The PC panels should be able to slide into the slots where needed to keep the frame loosely together.

  - Insert the two T nuts needed to join a corner of the frame, and line up the holes to do so. Once you have the two screws on the corner profile in place loosely, square up the 6 hole plate and tighten the four screws on the wide profile

  - Now, ensure the corner profile is square against the wide profile, and tighten the remaining two screws to keep it in place.

    - Repeat on the opposite side

  - Grab the last wide profile and two 6-hole plates, adding four nuts and screws and setting the plates up the same way you did on the side walls.

    - If, like me, you don't have 20+ drop in nuts, slide in a total of 24 nuts, 12 into each channel of the profile, so you have 16 extra to attach the front later on.

  - Repeat the process outlined in steps c, d, and e to join this last wide profile at the front of the frame

  - Use a large square or something with a 90 degree corner to check that the frame is squared up. Loosen, adjust, and tighten the screws on the corners as needed. Done!

- Ceiling Attachments

  - If your arena will have lights or cameras or anything else attached to the underside of the ceiling, now is the time to connect that all, and route wiring out a corner of the ceiling frame! The lights I got in particular happen to have slotted mounts that will work perfectly with T nut and screw and maybe a washer. However the wires will need to be extended to suit your particular setup.

  - Since everyone’s needs will be different, this will be left up to the user.

- Attach the Ceiling to the Frame

  - This will use:

    - All assembled frame parts

    - 4X 4-hole L corner brackets

  - If you have handles, now is a great time to attach them to the ceiling however you see fit.

    - You will need to pick an orientation for the ceiling as these will only fit on the 48” long profiles

  - Grab a pair of L plates, and insert two screws and nuts at the long side of the L. Slide these down into the frame corner profiles on a side of the frame so the short sides stick out the top.

    - Repeat on the opposite side

    - The sides chosen should depend on which sides you expect people to stand on when fighting - try not to have the ceiling central profile in line with the sides the competitors will be standing on or they will have an obstructed view.  I set mine up so competitors may stand at the front and back, with the ceiling’s wide profile parallel to the front/doorway, and L plates on the sides.

  - With the help of a friend, lift the ceiling up on top of the frame. Orient it so the 48” side profiles line up with the L brackets, and lower it carefully down onto the frame.

  - Insert two T nuts into each corner of the ceiling, using a T handle or long allen wrench to push them into position lined up with the holes on the L plates, and then insert screws to thread into them. Loosely tighten these.

    - Repeat on the three other corners

  - Done with the hard part!

- Install the Front Panels and Door

  - This will use:

    - Assembled frame

    - 6X rectangle 6-hole plates

    - 20X Wood screws and washers, ideally short enough not to protrude through the wood

    - 2X Hinges

    - Gate latch and matching latch bolt w/ mounting screws

    - Pilot drills sized for both wood screw sizes

    - Pencil to mark holes

  - In order to mark the holes on the bottom of the wood, you can make a jig from the 6-hole plates. Mark the holes as shown, two spaced just away from the inside edge and two spaces at least 3 inches from the outside edge, on both panels. Then pilot drill all holes.

    - The top edge holes will be marked and drilled in-place once the bottom is screwed on

  - Find or insert four sets of four T nuts in the front bottom wide profile, and loosely secure a 6-hole plate vertically to each with 4 screws, so the top two holes overlap where the wood will be screwed to.

  - Position one wood panel in the front of the frame lining up the pilot holes with the 6-hole plate holes, and secure with the four needed wood screws/washers to the two plates, ensuring they are square to the frame before tightening.

    - Repeat on the opposite side. Note, this should be mirrored as it's asymmetrical.

  - Insert six T nuts into the top front profile, three on each side, and loosely secure two 6-hole plates sideways with these

    - You may also want to insert any you need for your inches (in my case 3 per side) if you cannot roll or drop in more nuts later

  - Measure out a placement location for these plates, then fully tighten them to the profile. Now, mark, drill, and screw on the plates to the wood panel accordingly on both sides.

  - Grab your hinges. Add more T nuts (if not done before) and screws to loosely attach the tops of the hinges to the same profile. Again, you will need to measure and position these as you see fit for the door.

    - The hinge axis must be at or above the edge of the profile, or the front of the door will hit the profile on top when it swings open

  - Grab the wood piece for the door. You may need a shim or spacer underneath it so it becomes flush with the top of the frame and has a small gap at the bottom so it can swing open.

    - This lower gap is covered by the kick plates if yours are more than 2” tall. If not you might need to add a lip or something to prevent shrapnel from escaping under the door.

    - You also may want to cut a long strip of MDF about ¾” x ¾” x 45” to fill the gap between the back of the door and the back of the kickplate, and provide a softer stop for the door when it swings closed.

  - Mark and drill the holes for the door-side of the hinges, and secure them with screws

    - You may find some sanding or repositioning is needed to get the door to close consistently and smoothly. If there ends up being a large gap, you might want to add strips of material out in front of the door on both sides to prevent shrapnel from escaping.

  - Now, figure out where exactly you want the gate latch and latch bolt on the door and wall section. The latch must be mounted to the stationary wall so the bolt can swing out of it when the door is opened.

    - The relative heights of the latch and bolt are very critical, as are having them both parallel to the hinge axis, or the door may bind or the latch fail to close properly on the bolt. You may need to file the inside of the latch to get a looser fit, but do not remove so much material that the door can swing open with the latch closed.

  - Mark, drill, and screw on the latch and bolt. Adjust as needed for a consistent closing/locking action and ensure the door can always swing open when the latch is unlocked.

    - Using a standard gate latch is by no means the only locking mechanism that works well here, and it can be fiddly to position it well, but I strongly prefer a locking system that self-engages when the door is closed rather than one where you need to remember to lock it. This removes a huge potential safety hazard of leaving the box unlocked with a bot inside.

  - That’s it! You’re done! All that’s left is some cleanup work, wiring and cable management if you have any to do, and angling/positioning any lights and cameras you might have inside the box.

## Disassembly

- To collapse the arena, the ceiling will remain as one assembled piece, and the walls will need to be separated from it. The easiest way to disassemble the frame will be to remove the ceiling, then all four corner profiles:

  - Remove all T slot screws from the front of the arena that are attached to hinges or plates screwed to wood pieces, leaving wood screws in place. Carefully remove the wood pieces and place them somewhere out of the way.

    - Wood screws and generally not designed to be removed and reinserted several times, so avoid doing so if possible

  - Loosen all 8 of the screws on the L brackets on vertical profiles that the ceiling is connected to

  - Have a helper aid in lifting the ceiling straight up off the frame, and set it aside.

  - Lift out all three PC wall panels, and set them aside somewhere they won't get scratched/dirty.

  - Loosen all screws at the corners of the frame attached to each vertical corner profile, and lift these straight up to remove them as well.

  - The arena should now be able to be propped up flat against a wall, as the subfloor and ceiling may rest flat on one another, and then the wood and polycarbonate and four corner profiles can be laid carefully on top of those.

  - To reassemble, reverse these steps.

## Modifying the Design

If you choose to resize the arena or wish to fight larger than 1lb bots, you may wish to refer to the following:

|  | Slot Width |  | Slot Depth |  |  |
| --- | --- | --- | --- | --- | --- |
| 80/20 Profile Series | Inch | mm | Inch | mm | PC Max Thickness |
| 10 Series (1.0") | 0.256 | 6.502 | 0.323 | 8.204 | 6mm or 1/4" |
| 15 series (1.5") | 0.320 | 8.128 | 0.484 | 12.294 | 8mm |
| 20 series (20mm) | 0.207 | 5.260 | 0.244 | 6.200 | 5mm or 3/16" |
| 25 series (25mm) | 0.256 | 6.500 | 0.319 | 8.100 | 6mm or 1/4" |
| 30 series (30mm) | 0.320 | 8.140 | 0.368 | 9.340 | 8mm |
| 40 series (40mm) | 0.320 | 8.130 | 0.485 | 12.320 | 8mm |
| 45 series (45mm) | 0.399 | 10.130 | 0.512 | 13.000 | 10mm or 3/8" |

The above table should aid in selecting the proper size panels and proper length hardware.